# USBCFTEST Utility

USBCFTEST utility is an utility to send the commands to SanDisk ImageMate SDDR-31 Compact Flash Reader. ImageMate SDDR-31 connects to personal computers through USB (Universal Serial Bus) I/f. USBCFTEST utility runs under Microsoft Windows-98 operating system.

USBCFTEST utility can be run only on systems, which have Imagemate SDDR-31 configured (device drivers already installed) in system. To find that ImageMate software is installed in system, you can plug ImageMate SDDR-31 through USB connector, as a result SanDisk logo should be visible in taskbar.

Now Start the “USB Compact Flash Test”. Now select the “command” through ToolBar or through View menu. Depending upon level of commands to be sent, select one of the following two tabs:

- Basic Commands (Any revision of the firmware in SDDR-31 will work with basic commands.)

- Low Level Commands (The revision of the firmware in SDDR-31 must be 10 or higher to run.)

Basic Commands: Basic commands interface provides high level functions to ImageMate SDDR-31. To run most of these commands, you need to have Compact Flash media inserted in Imagemate SDDR-31, before running this utility. The Imagemate is selected through Adapter, Target and LUN ID. Each block is defined as 512 byte of data. The unit of operation for these commands is in block (512 byte sector). The following commands are part of Basic Command set. All commands status is returned in “result” field. For commands with data transfer functions, the “FileName” must be specified first by clicking “Browse” button. The starting and ending block should be specified in the “Block” box.

- Reset CF: Not Implemented.

- Read Block(s): This command reads the Block specified by “Start” & “End” from the compact flash and data read is stored in file specified in “FileName” field.

- Write Block(s): This command writes the Block specified by “Start” & “End” to the compact flash and data written is read from the file specified in “FileName” field.

- Compare Block(s): This command compares the Block specified by “Start” & “End” from the compact flash and data read is compared to the data stored in file specified by “FileName” field.

- Erase Block(s): This command clears (all zero) the contents of the data Block specified by “Start” & “End” from the compact flash.

- Write Pattern(s): This command writes the Block specified by “Start” & “End” to the compact flash and data written is specified by Pattern field. The pattern is repeated at interval of 32 bytes. The pattern data (byte unit) can be specified either in hex-decimal or numeral format. The number starting with 0x is considered as hex-decimal number. The maximum value specified can be 255 (hex: 0xff).

- Compare Pattern(s): This command compares the Block specified by “Start” & “End” of the compact flash from data specified by Pattern field. The pattern is repeated at interval of 32 bytes. The pattern data (byte unit) can be specified either in hex-decimal or numeral format. The number starting with 0x is considered as hex-decimal number. The maximum value specified can be 255 (hex: 0xff).

- Invert Pattern(s): This command inverts (1’s complement) the data specified in Pattern field.

- Read Binary File: This command reads the binary file specified by “FileName” in hex format.

- View Log: This command reads the status log of all commands executed so far.

- Stop Logging: This command stops the logging to LOG file.

- No Operation: This command performs no operation.

Low Level Commands: This command sends the low level SCSI commands to ImageMate SDDR-31. The ATA commands are directly sent to Compact Flash media, and are not interpreted by Imagemate SDDR-31. Each command is executed “Repeat Count” times sequentially. The command can be specified to ‘stop” after an execution of it returns failure. The direction of data is specified independently for SCSI & ATA commands. The data field of command is stored or loaded from the file specified by “FileName” field. The data length field is specified by “Data Length”. The result of all commands are reported in “Status” field. For commands with data transfer functions, the “FileName” must be specified first by clicking “Browse” button. The “Data Length” and “Data Direction In” must be correct.

- Reset CF: This command is not implemented.

- Execute SCSI Command: This command sends the SCSI command specified by CDB00-15. All SCSI commands are sent to ImageMate SDDR-31. Following the command phase, data is sent if “Data Length” is non-zero. The direction of data transfer depends upon “Data Direction In” (if selected, the direction is from ImageMate to Personal Computer) specified by SCSI Command. The data is read or written to / from the file specified by “FileName”.

- Execute ATA Command: This command sends the ATA Task File Register to ImageMate SDDR-31. Compact Flash media executes all ATA commands. Following the command phase, data is sent if “Data Length” is non-zero. The direction of data transfer depends upon “Data Direction In” (if selected, the direction is from ImageMate to Personal Computer) specified by ATA Task File Register. The data is read or written to / from the file specified by “FileName”. After the command is executed, The compact flash ATA registers are updated in read field of ATA Task File Registers. ATA commands can be sent in ATA format, if specified by “LBA Mode”.

- Read Binary File: This command reads the binary file specified by “FileName” in hex format.

- View Log: This command reads the status log of all commands executed so far.

- Stop Logging: This command stops the logging to LOG file.

- No Operation: This command performs no operation.