6831 Oxford Street, St. Louis Park, Minnesota 55426, U.S.A.

# Technical Support   Telephone: (952) 924-1123   Fax: (952) 924-1122

Julie Ruettimann:  (952) 924-1121      EMAIL: julie.ruettimann@shadin.com

## Troubleshooting the Floscan Transducer

(No Fuel Flow)

- The Floscan transducer (Shadin P/N 68050X) has three wires coming out of the top – Red, White, and Black.

- The Red wire should have 8 to 12 volts on it with respect to the black wire.

- If 8 to 12 volts is not present, check the wiring back to the Fuel Flow Indicator.  If the wiring is good, the Fuel Flow Indicator needs repair.

- The White wire should be disconnected at the splice between the transducer pigtail and the wiring harness going back to the Fuel Flow Indicator.  5 volts should be present on the wire that goes back to the Fuel Flow Indicator.

- If the 5 volts is not present, check the wiring back to the Fuel Flow Indicator.  If the wiring is good, then the Fuel Flow Indicator needs repair.

- If the 5 volts is present, proceed to step 7.

- Tap the wire with the 5 volts on it very rapidly to ground.  A few tenths of fuel flow should be observed on the Fuel Flow Indicator.

- If a few tenths of fuel flow is observed, then the transducer needs to be replaced.