# Post Installation Troubleshooting Guide

## Display Reads	Cause

Shadin Cannot read the Nav Reciever’s serial data information.  Verify wiring

### L-Bad

Shadin not receiving signal from Nav reciever.  If this is the initial powerup 				after a new installation, it may take as long as 20 minutes of searching before the 			Shadin system “locks on” to what GPS/Loran system it is listening to.  					If the problem continues, verify your wiring and make sure the GPS/Loran is 				configured properly for the Shadin system.

L-OFF

Stands for “Loran Search”.   Shadin is searching for correct signal to lock onto.  				Doublechack for proper  wiring (if wiring RS422/485, try reversing the RS422 + 			an - ).  If the display is alternating between LS3, LOFF, LS4, LOFF etc, see L-				OFF above.

LS-1,2,3

Loran or GPS signal understood.  No meaningful numbers will appear until the				Nav has acquired and a destination is selected (final waypoint).  Other 					requirements are 26Kts of groundspeed and operational fuel flow.

### - - - - - -

Shadin system has lost it’s memory and will need to be reprogrammed.  This can 			be accomplished in the field without sending the unit back to the factory.  Call 				 the Shadin Technical Support Center for reprogramming instructions.

E-1

Rambad

Read-only memory has been corrupted.  The unit must be returned to the factory 			for service.

Stands for Channel 1 or 2 is inoperative.  The circuit to the processor board 				(internal) is open.  This unit must be sent back to the factory for service.

CH1Bd

Only found on older Shadin fuel systems.  If message disappears after initial 				warmup, then this is only a “check-sum” error and will not interfere with the 				operation of your fuel system.  If message does not disappear after start-up, the 				unit will need to be sent back to the factory for service.

ERR1

This message will only appear on the Digidata.  If after 20 minutes the message 				does not go away,  it means that the internal heater circuit for either the PALT or 			IAS transducer has failed the warm-up calibration routine.  Return the unit to 				factory for service.

### Warm-UP

Check for proper power input (14-28VDC) from A/C circuit breaker and a 				proper  ground.

### Display blank

### No fuel flow display

Check your fuel flow transducer signal wire (white wire) for proper 0-5VDC 				square wave signal going into correct pin of the Shadin indicator.  Additionally, 				check for 8-12VDC on the power line (red wire) and for proper ground (black 				wire)

Shadin Technical Support hours are 9AM to 5PM CST, M-F

Phone:800-328-0584 or 612-924-1141. Fax:1122  Email: techsupt@Shadin.com

Tips to review when installing a Shadin Fuel Flow system

When installing a “bare” Floscan transducer (Shadin P/N 68050X), make sure the transducer is in a horizontal position with the 3 wires 	pointing directly up.  Failure to follow this tip will result in premature failure of the transducer and erratic readings.

Remember: Horizontal, with wires straight up!

- When tightening the AN fittings on the Floscan transducer (Shadin P/N 68050X), the maximum torque pressure to be used is 15ft/Lbs!

- Do not ground the black wire (Ground) at the engine block.  Carry the Ground back the Shadin fuel flow indicator.  Failure to follow this suggestion may result in inaccurate fuel readings (because of differences in Ground potential).

- When installing a Shadin Fuel system, it is recommended that the installer use 3 conductor, 22 gauge, shielded wire to run between the transducer(s) and the Shadin fuel indicator.  The shield should be terminated at the indicator end, not at the transducer end.   Failure to follow this recommendation will result in the possibility of a 10-16% error (on the high side) caused by electronic “noise” on board the aircraft.

Fuel Flow Transducer

Shadin Indicator

Power (red)

Signal (White)

Ground (Black)

Do not terminate the shield at this end!

#### Inaccurate fuel flow readings

On reciprocating engines (Continental/Lycoming) using

Shadin Fuel transducer P/N 680501

- See items # 1, 3, & 4 as they apply

- No 90 degree fittings within 3” of the inlet side of the transducer.  This creates cavitation in the fuel and will result in inaccurate fuel flow readings.

- Readings are 50% of what they should be.  Check that your fuel flow transducer has not been installed backwards.

8)	Consistent high fuel flow reading (on Lycoming powered product).

Check # 4 first, if that checks out, then………

If this aircraft has an auxiliary pump, try turning the Aux pump on next time 	the aircraft is being flown in “cruise”.  If the fuel flow drops (after about 	minute), this is an indication that air is leaking into the fuel line upstream of 	the transducer.  Check gaskets for stains (this has only been observed in 	Lycoming powered aircraft).  It is possible that no staining will be observed 	because the pressure is great enough to pull air into the system but not let fuel 	escape.  If the customer choses not to replace the gaskets go to # 9 of this section 	for K-Factor setting instructions for more accurate fuel flow.

9)	Aircraft gauge formula to correct minor fuel flow errors:

Total Fuel Used (from fill-up to fill-up)

##### As displayed under “fuel Used”

on Shadin Indicator			=	Correction multiplier

##### Actual fuel

##### filled up after the flight

Correction 	X 	Existing	 = 	New Corrected K-Factor

Multiplier 		K-Factor

- No fuel flow on right engine (in twin applications).

Make sure the unit is properly programed for twin application.  If programming 	is not the cause, make sure the transducer power, signal and ground lines are 	correctly wired.  Remember that in some of the systems left and right power is 	shared and left and right grounds are shared.

Fuel flow inaccuracies when using the larger Shadin fuel flow transducers (P/N 6605XX-XX) (for turbine and radial engine applications)

###### There are 3 pins associated with the coil top

Pin

A	Regulated 12 VDC from the indicator out to the transducer (to set up the 			magnetic field)

B	Signal (0-5 VDC Square wave)

C	Ground

The indicator in the cockpit comes pre-programmed to match the K-Factor 	of the 	transducer.   Remember:  Transducers are individual, hand made items.  	When 	they go through the calibration procedure, whatever K-Factor they flow 	out at is what they will be, there is no changing this 	number.  The indicator in 	the cockpit is the system which can be changed to any k-factor setting.

11)	Fuel reading inaccurate (always check this first)

For newer systems (after 1991), make sure the indicator is calibrated to the correct 	K-Factor by holding in the Enter/Test button on the indicator. 88888 will flash 	across the screen, release the button.  The word “Good” will appear.  The next 	value(s) which appear will be what the K-Factor is set to (i.e. 41.2 = 41,200 	pulses).  Compare this number with the red calibration tag on the side of the 	6605XX transducer.  They should be the same.

12)	Fuel indicator reading erratic or high.  (also see items 3, 4, 7 & 10)

Try re-clocking the transducer 90 degrees and retightening.  As the black 	magnetic pick-up coil (which is attached to the top of the stainless steel 	transducer) is susceptible to EMI, changing the orientation will sometimes 	make the problem go away.  The Shadin fuel flow transducer will perform 	the 	same regardless of which direction the coil top is pointing.  If this does not 	solve the problem, purchase Shadin P/N 610507 (straight) or P/N 610508 	(right 	angle) magnetic shield, this will protect the coil top from EMI.

Shadin P/N 610507

13)	If you are not getting any flow reading on your indicator.

Disconnect, inspect, and re-connect the MS connector attached to the top of the 	transducer.  You may even try checking continuity of the 3 wires to the cockpit.

14)	If you are not getting any flow reading on your indicator.

Remove the 4 allen screws holding the black coil-top in place.  With the		indicator power on, wave a magnetic screwdriver back and forth across the 	face of the coil top.  If there is no flow indicated, either replace the coil top 	with a new one (Shadin P/N 690501 – straight coil top or Shadin P/N 690502 	– right angle coil top).  The coil top is a part which can be re-built, so if you 	chose that option, contact the Technical Support Department.

If there is flow indicated, there is a problem with the bearings or turbine inside 	the transducer and will have to come back to Shadin for service

Miscellaneous problems seen in the past…….

1)	Interference with indictor caused by incorrect cutting and stowing of an 			unused RS232/422 transmit line.  Make sure that you do not coil this line 			as it will set up a magnetic field (keep straight).

- Ground Loop.  The shield on the shielded wire has been has been terminated at both ends (or sometimes in the middle!).  This happens quite often when carrying the shield through a bulkhead connector.  The installer must carry through the bulkhead connector with a dedicated pin.  What happens sometimes is that the installer will terminate the shield to the mounting screws of the MS bulkhead connector, thus going to ground and creating a “ground loop effect”.	 Remember :  The shield on the Shadin system can only be terminated at the indicator to reduce the antenna (or ground loop) effect.

Programming Group 2 configuration

The Shadin fuel flow indicators (Miniflo/Microflo/Digiflo) have 2 different configurations.

Group 1:	Requires the installer to remove the indicator from the cockpit and reset 			operation switches prior to recalibrating/reprogramming the unit.  Contact 			Technical Support if Group 1 reprogramming required.

Group 2: 	Settings can be done from the face of the indicator without instrument 			removal.

GROUP 1				GROUP 2

Left & Right K-Factors			Output Type

Fuel Units				Loran Input

Single or Twin				Endurance Warning Time

Low Flow Cutoff				Filter Type							Low fuel level

+ Plus all Group 2 functions		Ignore Loran Warning

- Access Group 2 by holding in the enter/test button.  The system will go through an internal self test then begin to count down from 15.  Once the word “ENT” appears, you  may release the button.  You are now in Group 2 entry mode.

- The USED/REM toggle switch is used to page through the options.  The ADD/FULL switch is used to scroll through the selections (within each option)

- Once the desired selections have been made, press and hold the ENTER/TEST button.  The system will count down from “5”.  Release the button when the

word “SET” appears.  The new configurations are now in effect.

OPTION	SELECTION				OPTION	SELECTION

o		Loran/GPS Output			1	LORAN/GPS Input

0 =  Off (all Northstar Products)			0 = OFF (without knob)

1 = Bendix/King KLN series			1 = ON (with knob)

2 = Shadin Airdata

3 = Arnav/Magellan

4 = Trimble			d	Endurance Warning Time

5 = Generic/Garmin				0 = 45 Minutes

1 = 5 Minutes

F		Filter Type					2 = 10 Minutes

0 = Injected/Turbine engines			3 = 20 Minutes

1 = Carburated engines			4 = 30 Minutes

U		Ignore Loran Warning			S	Low fuel level warning		0 = No (Default – all but Foster systems)		(display in units)

1 = Yes Old Foster – Call Shadin

.